# Traditional vs Coverged Networks

- On old traditional networks, data, voice and video had their own separate network infrastructure and did not impact each other

- On modern networks, data, voice and video run over the same shared infrastructure

- This enables cost savings and advanced features for voice and video

- Data, voice and video are all fighting for the same shared bandwidth

# Quality Requirements for Voice and Video

- Voice and traditional standard definition video packets must meet these recommended requirements to be an acceptable quality call:

  - Latency (delay) ≤ 150 ms

  - Jitter (variation in delay) ≤ 30 ms

  - Loss ≤ 1%

- These are one way requirements, meaning a packet sent from a phone in HQ has 150ms to reach the phone in the branch, and vice versa

- HD video has stricter requirements

# FIFO First In First Out

- Whenever congestion is experienced on a router or switch, packets are sent out in a First In First Out (FIFO) manner by default

- Congestion can be experienced whenever it is possible for packets to come in quicker than they can be sent out

# Congestion Examples

- Traffic is going left to right from the HQ to the branch

- The WAN edge router has a FastEthernet interface on the inside LAN interface and an E1 interface on the outside WAN interface

- Traffic destined for the branch comes in on the LAN interface at a rate

- lower than 2 Mbps

- Packets can be sent out immediately as they arrive – there is no congestion

- Traffic destined for the branch comes in on the LAN interface at a rate

- higher than 2 Mbps

- Packets are arriving faster than they can be sent out

- Packets wait in the queue to go out

- Packets are sent out FIFO in the order they were received

# Effects of Congestion

- Congestion causes delay to packets as they wait in the queue

- As the size of the queue changes it causes jitter

- There is a limit to the size of the queue. If a packet arrives when the

- queue is full the router will drop it

- Voice and video calls (and applications) will be unacceptable quality if they do not met their delay, jitter and loss requirements

# How to Mitigate Congestion

- Add more bandwidth (this costs money)

- Use Quality of Service techniques to give better service to the traffic

- which needs it

# Congestion Example with QoS Queuing

- Traffic destined for the branch comes in on the LAN interface at a rate higher than 2 Mbps

- Packets are arriving faster than they can be sent out

- Packets wait in the queue to go out

- The router recognises the voice packets and moves them to the front of

- the queue to minimise their delay

# Effects of QoS Queuing

- QoS queuing can reduce latency, jitter and loss for particular traffic

- The original driver for QoS was Voice over IP but it can also be used to give better service to data applications

- If you’re giving one type of traffic better service on the same link you started with, the other traffic types must get worse service

- The point is to give each type of traffic the service it requires

- QoS queuing is not a magic bullet and is designed to mitigate temporary periods of congestion. If a link is permanently congested the bandwidth should be increased

# Classification and Marking

- For a  router or switch to give a particular level of service to a type of traffic, it has to recognise that traffic first

- Common ways to recognise the traffic are by COS (Class of Service) marking, DSCP (Differentiated Service Code Point) marking, an Access Control List, or NBAR (Network Based Application Recognition)

## Layer 2 Marking - CoS Class of Service

- There is a 3 bit field in the Layer 2 802.1q frame header which is used to carry the CoS QoS marking

- A value of 0 – 7 can be set. The default value is 0 which is designated as Best Effort traffic

- CoS 6 and 7 are reserved for network use

- IP phones mark their call signalling traffic as CoS 3 and their voice payload as CoS 5

## Layer 3 Marking - DSCP

- The ToS Type of Service byte in the Layer 3 IP header is used to carry the DSCP QoS marking

- 6 bits are used which gives 64 possible values. The default value is 0 which is designated as Best Effort traffic

- IP phones mark their call signalling traffic as 24 (CS3) and their voice payload as 46 (EF)

- There are standard markings for other traffic types, such as 26 (AF31) for mission critical data, and 34 (AF41) for SD video

## Recognising Traffic with an ACL

- An Access Control List can be used to recognise traffic based on its Layer 3 and Layer 4 information

- For example SSH traffic going to and from the router 10.10.100.10 on TCP port number 22

## Recognising Traffic with NBAR

- NBAR (Network Based Application Recognition) can be used to recognise traffic based on its Layer 3 to Layer 7 information

- Signatures can be downloaded from Cisco and loaded on your router which recognise well known applications

# Congestion Management

- Queuing can be used to manage congestion on routers and switches

- CBWFQ (Class Based Weighted Fair Queuing) gives bandwidth guarantees to specified traffic types

- LLQ (Low Latency Queuing) is CBWFQ with a priority queue

- Traffic in the priority queue is sent before other traffic

# MQC Modular QoS CLI

- Cisco QoS configuration uses the MQC Modular QoS CLI

- It has 3 main sections

- Class Maps define the traffic to take an action on

- Policy Maps take the action on that traffic

- Service Policies apply the policy to an interface

(This config isn’t required on the CCNA)

# Shaping and Policing

- Traffic Shaping and Policing can be used to control traffic rate.

- They both measure the rate of traffic through an interface and take an action if the rate is above a configured limit.

- Traffic shaping buffers any excess traffic so the overall traffic stays within the desired rate limit.

- Traffic policing drops or re-marks excess traffic to enforce the specified rate limit.

- Classification can be used to allow different rates for different traffic types.

(This config isn’t required on the CCNA)