# IOS Security

- When a Cisco router or switch is received from the factory no security is configured

- You can access the command line via a console cable with no password required

- One of the first tasks is to configure security to ensure that only authorised administrators can access the device

IOS Command Hierarchy

- > = User Exec mode

- # = Privileged Exec moe (“Enable”)

- (config)# = Global Confuration mode (“Configure Terminal”)

- (config-if)# = Interface Configuration mode (“Interface x”)

# Basic Line Level Security

- Minimal password security can be configured through the use of static, locally defined passwords at three different levels:

  - Console line – accessing User Exec mode when connecting via a console cable

  - Virtual terminal VTY line – accessing User Exec mode when connecting remotely via Telnet or SSH Secure Shell

  - Privileged Exec Mode – entering the ‘enable’ command

- The levels can be used independently or in combination with each other.

- They can use the same or different passwords.

## Basic Console Security

- Only one administrator can connect over a console cable at a time so the line number is always 0.

- ‘Login’ with no following keywords requires the administrator to enter the password configured at the line level to log in

  - R1(config)#line console 0

  - R1(config-line)#password Flackbox1

  - R1(config-line)#login

Switch Management IP Address

- A Layer 2 Switch is not IP routing aware

- It does however support a single IP address for management

- A default gateway also needs to be configured to allow connectivity to other subnets

Switch(config)# interface vlan 1

Switch(config-if)# ip address 192.168.0.10 255.255.255.0

Switch(config-if)# no shutdown

Switch(config-if)# exit

Switch(config)# ip default-gateway 192.168.0.1

## Basic Telnet Security

- An administrator can use Telnet to connect to the CLI of a router or switch remotely over an IP connection

- IOS devices do not accept incoming Telnet sessions by default

- An IP address and virtual terminal VTY line access must be configured

- Multiple administrators can connect at the same time. Lines are allocated on a first come first served basis

- If all configured lines are in use then additional administrators will not be able to login

R1(config)#line vty 0 15

R1(config-line)#password Flackbox2

R1(config-line)#login

## Exec Timeout

- An administrator will be logged out after 10 minutes of inactivity by default. This applies to both the console and VTY lines

- You can edit this value with the exec-timeout command

- no exec-timeout or exec-timeout 0 allows an administrator to stay logged in indefinitely

R1(config)#line con 0

R1(config-line)#exec-timeout 15

R1(config)#line vty 0 15

R1(config-line)#exec-timeout 5 30

## Securing VTY Lines with Access Lists

- You can apply an Access List to control access to the VTY lines

- This can be used to limit Telnet and SSH access to only your administrator workstations

R1(config)#access-list 1 permit host 10.0.0.10

R1(config)#line vty 0 15

R1(config-line)#login

R1(config-line)#password Flackbox3

R1(config-line)#access-class 1 in

## Basic Privileged Exec Security

- When you connect over the console or a VTY line you will land at the User Exec prompt which has a very limited set of commands available

- To get superuser access you use the ‘enable’ command to invoke Privileged Exec mode

- This can be secured with a password

- Disadvantage: enable password can be viewed in the show run config

R1(config)#enable password Flackbox3

### Enable Secret

- An enable secret performs the same function as the enable password

- The enable secret is always shown in an encrypted format in the running configuration

- If both an enable password and enable secret are configured, the enable secret supersedes the enable password which is no longer used

- Best practice is to configure an enable secret but not an enable password

### Encrypting Passwords

Line level passwords can also be viewed in plain text in the running configuration by default.

### Service Password-Encryption

- The service password encryption command encrypts all passwords in the running configuration

- It is best practice to enable this

R1(config)#service password-encryption

# Username Level Security

- More granular security can be provided by configuring individual usernames and passwords for different administrators

R1(config)#username admin1 secret Flackbox1

R1(config)#username admin2 secret Flackbox2

R1(config)#line console 0

R1(config-line)#login local (use local usernames)

R1(config)#line vty 0 15

R1(config-line)#login local

## Privilege Levels

- There are 16 privilege levels of admin access (0-15) available on a Cisco router or switch

- Usernames can be assigned a privilege level. The default level is 1.

- You can also configure different passwords for direct access to the different privilege levels

- Each available command in IOS can be assigned a privilege level. An administrator must be logged in with that privilege level or higher to run the command

- By default, three levels of privilege are used - zero, user, and privileged. All commands are at one of these three levels by default

- Zero-level access allows only five commands—logout, enable, disable, help, and exit.

- User level (level 1) provides very limited read-only access to the router. When you enter User Exec Mode you’re at Privilege Level 1 by default

- Privileged level (level 15) provides complete control over the router. When you enter Privileged Exec Mode with the ‘enable’ command you’re at Level 15 by default

R1(config)#username admin1 secret Flackbox1

R1(config)#username admin2 privilege 15 secret Flackbox2

R1(config)#line console 0

R1(config-line)#login local

R1(config)#line vty 0 15

R1(config-line)#login local

## Configuring Command Privilege Levels Example

Only admin2 has superuser privileges

R1(config)#username admin1 secret Flackbox1

R1(config)#username admin2 privilege 15 secret Flackbox2

R1(config)#username admin3 privilege 5 secret Flackbox3

Change command privilege level. Now also admin3 can execute show run conf

R1(config)#privilege exec level 5 show running-config

R1(config)#enable secret secret1 (sets password for privilege level 15)

R1(config)#enable secret level 5 secret2 (sets password for privilege level 5)

# Telnet vs SSH

- All Telnet communications cross the network in plain text

- If somebody sniffs the traffic using a tool such as Wireshark they can see all the commands you enter including your username and password

- All SSH Secure Shell traffic is encrypted

- If somebody sniffs the traffic they cannot read it

- Best practice is to disable Telnet and only allow SSH for administrator CLI access

Enable SSH

- A digital certificate with a key length of at least 768 bits must be generated to enable SSH encryption

Disable Telnet

- VTY lines are used for both Telnet and SSH connections

- Access is allowed for both by default

- A username is required for SSH access (line level passwords are not supported)

R1(config)#username Flackbox secret Flackbox1

R1(config)#line vty 0 15

R1(config-line)#transport input ssh (telnet not added)

R1(config-line)#login local (use local usernames)

R1(config-line)#exit

R1(config)#ip ssh version 2 (limit SSH to v2)

# AAA Server

- Configuring line level security or local usernames on each device has a serious scalability limitation

- If a password has to be added, changed or removed it needs to be done on all devices

- An external AAA server can be used to centralise this instead

- Multiple AAA servers can be implemented for redundancy

- AAA servers provide Authentication, Authorization and Accounting.

- Authentication verifies somebody is who they say they are. This is most commonly achieved with a username and password.

- Authorization specifies what a particular user is allowed to do, such as running a particular command.

- Accounting keeps track of the actions a user has carried out.

- Authorization and Accounting are optional. Authentication is mandatory if Authorization and/or Accounting are used.

## RADIUS and TACACS+

- The protocols which are used for AAA services are RADIUS and TACACS+

- Both are open standards, although vendors may add their own proprietary extensions

- Many vendor’s AAA servers support both protocols

- RADIUS is commonly used for end user level services, such as VPN access

- TACACS+ is commonly used for administrator access on Cisco devices as it has more granular authorization capabilities

## Cisco AAA Servers

- Cisco’s AAA server is the Identity Services Engine (ISE)

- They also offered the Access Control Server (ACS) for a long time but it is now end of sale

## Active Directory Integration

# OR

# RADIUS/TACACS+ Configuration

## Old RADIUS Configuration

R1(config)#username BackupAdmin secret Flackbox1 (configure a local user in case connectivity to the AAA server is lost)

R1(config)#aaa new-model

R1(config)#radius-server host 10.10.10.10 key Flackbox1

R1(config)#radius-server host 10.10.10.11 key Flackbox2

R1(config)#aaa group server radius FB-RG (optional)

R1(config-sg-radius)#server 10.10.10.10

R1(config-sg-radius)#server 10.10.10.11

R1(config)#aaa authentication login default group radius local

(Use all RADIUS servers) OR:

R1(config)#aaa authentication login default group FB-RG local

(Use servers in specified group)

## New RADIUS Configuration

R1(config)#radius-server host 10.10.10.10

Warning: This CLI will be deprecated soon. Please move to radius server <name> CLI.

R1(config)#aaa new-model

R1(config)#radius server Server1

R1(config-radius-server)# address ipv4 10.10.10.10

R1(config-radius-server)# key Flackbox1

R1(config)#radius server Server2

R1(config-radius-server)# address ipv4 10.10.10.11

R1(config-radius-server)# key Flackbox2

R1(config-radius-server)#aaa group server radius FB-RG

R1(config-sg-radius)# server name Server1

R1(config-sg-radius)# server name Server2

R1(config-sg-radius)#aaa authentication login default group FB-RG local

## Old TACACS+ Configuration

R1(config)#username BackupAdmin secret Flackbox1

R1(config)#aaa new-model

R1(config)#tacacs-server host 10.10.10.10 key Flackbox1

R1(config)#tacacs-server host 10.10.10.11 key Flackbox2

R1(config)#aaa group server tacacs+ FB-TG

R1(config-sg-tacacs+)#server 10.10.10.10

R1(config-sg-tacacs+)#server 10.10.10.11

R1(config)#aaa authentication login default group FB-TG local

## New TACACS+ Configuration

R1(config)#tacacs-server host 10.10.10.10

Warning: This CLI will be deprecated soon. Please move to tacacs server <name> CLI.

R1(config)#username BackupAdmin secret Flackbox1

R1(config)#aaa new-model

R1(config)#tacacs server Server1

R1(config-server-tacacs)# address ipv4 10.10.10.10

R1(config-server-tacacs)# key Flackbox1

R1(config)#tacacs server Server2

R1(config-server-tacacs)# address ipv4 10.10.10.11

R1(config-server-tacacs)# key Flackbox2

R1(config-radius-server)#aaa group server tacacs+ FB-TG

R1(config-sg-tacacs+)# server name Server1

R1(config-sg-tacacs+)# server name Server2

R1(config-sg-tacacs+)#aaa authentication login default group FB-TG local

# Best Practices

## Login and Exec Banners

- Messages can be displayed in the CLI before and/or after an administrator logs in to a Cisco IOS device

- This is most commonly used to display security warnings

## Disable Unused Services

- It is best practice to disable unused services

- This reduces the attack surface and also the load on the device

- HTTPS is sometimes used by GUI administration tools but HTTP should be disabled

- CDP should also be disabled in highly secure environments

R1(config)#no ip http server

R1(config)#no cdp run

## Time Synchronisation - NTP

- All servers and infrastructure devices in your network should be synchronised to the same time

- This aids in troubleshooting as logs will report the correct time that events occurred

- It is also required by several security features such as Kerberos authentication and digital certificates

NTP Network Time Protocol

- Servers and infrastructure devices can use their own internal clock or synchronise with an external NTP server

- An NTP server should be used to ensure all devices have the same time

- A Cisco router can function as an NTP server and/or client